# SCOPE OF WORK (SOW)

# Farm Automation System

## Climate Control & Nutrient Management

Project: Farm Automation System
Client: Monnet Group
Prepared By: Integrated Hydroponics India Pvt. Ltd. (InHydro)

# 1. Project Overview

Integrated Hydroponics India Pvt. Ltd. (InHydro) proposes the design, development, supply, installation, programming, testing, and commissioning of a Microcontroller-Based IoT Automation System for climate control and nutrient management of the farm.

The automation system is designed to maintain optimum environmental conditions by continuously monitoring field parameters and automatically controlling various equipment such as fans, cooling pads, foggers, air circulation fans, irrigation pumps, nutrient management systems, and overhead sprinklers.

The system shall operate automatically based on configurable setpoints while providing manual override, safety interlocks, cyclic timer operations, and future IoT expansion capabilities.

# 2. Objectives

The objectives of the automation system are:

- Maintain optimum greenhouse temperature.

- Maintain desired humidity levels.

- Reduce manual intervention.

- Improve crop health and consistency.

- Optimize water and nutrient utilization.

- Enable real-time monitoring and automation.

- Provide scalable and future-ready IoT-based control architecture.

# 4. Climate Control Automation

The climate control system shall continuously monitor temperature and humidity inside the farm and automatically regulate environmental conditions.

## 3.1 Fan Control

The fans shall operate automatically based on configurable temperature setpoints.

### Temperature Control Logic

### Low Temperature Setpoint

- All fans shall remain OFF.

### Medium Temperature Setpoint

- 50% of the installed fans shall operate.

### High Temperature Setpoint

- All fans shall operate simultaneously.

The switching sequence shall be automatic and configurable through the controller settings.

## 3.2 Cooling Pad Automation

The cooling pad water circulation motor shall operate automatically in conjunction with temperature control.

### Operating Logic

When the temperature reaches the configured Maximum Temperature Setpoint:

- Cooling pad water pump shall receive an ON command.

- The pump shall operate through a configurable cyclic timer.

- User shall be able to configure:

  - Pump ON Time

  - Pump OFF Time

  - Repeat Cycle

### Humidity Interlock

While the cooling pad pump is operating, the controller shall continuously monitor humidity.

If humidity reaches the configured Maximum Humidity Setpoint:

- Cooling pad pump shall stop immediately.

- Pump operation shall resume only after humidity falls below the permissible limit and the temperature demand still exists.

### Recommendation

InHydro recommends configurable setpoints for:

- Minimum Temperature

- Medium Temperature

- Maximum Temperature

- Minimum Humidity

- Maximum Humidity

This provides flexibility for different crops and seasonal conditions.

## 3.3 Fogger Automation

Humidity shall be maintained automatically using the fogger system.

### Operating Logic

When humidity falls below the configured Minimum Humidity Setpoint:

- Fogger shall start automatically.

The fogger shall operate through a programmable cyclic timer.

The controller shall continuously monitor humidity.

Once humidity reaches the configured Maximum Humidity Setpoint:

- Fogger shall stop automatically.

User configurable parameters:

- Fogger ON Time

- Fogger OFF Time

- Humidity Setpoints

## 3.4 Air Circulation Fan (ACF)

The Air Circulation Fans (ACF) shall operate independently using a programmable cyclic timer.

User adjustable parameters:

- ON Duration

- OFF Duration

- Enable/Disable Schedule

The cycle shall continuously repeat during the operating period.

## 3.5 Overhead Sprinkler Automation

The overhead sprinkler system shall operate through a programmable cyclic timer.

The controller shall provide:

- ON Time

- OFF Time

- Number of Cycles

- Manual Override

# 4. Nutrient Management Automation

The nutrient management system shall continuously monitor:

- Electrical Conductivity (EC)

- pH

## 4.1 EC Control

The controller shall continuously compare the measured EC value with the configured EC setpoint.

### High EC Condition

If the measured EC exceeds the configured setpoint:

- Plain water solenoid valve shall open automatically.

- Fresh water shall dilute the nutrient solution.

- The controller shall continuously monitor EC.

- Once the desired EC value is achieved, the solenoid valve shall close automatically.

## 4.2 Nutrient Tanks

Provision shall be available for:

- Nutrient Tank A

- Nutrient Tank B

Future automatic nutrient dosing can be integrated without major modification.

## 4.3 pH Monitoring

The automation system shall continuously monitor pH values.

Provision shall be available for future automatic pH correction using dosing pumps.

# 5. Irrigation Automation

The irrigation system shall operate on a timer-based schedule.

The irrigation controller shall provide:

- Daytime Irrigation Scheduling

- Multiple Irrigation Cycles

- Adjustable Irrigation Duration

- Adjustable Start Time

- Manual Irrigation Mode

The irrigation schedule shall primarily operate during daytime as configured by the user.

# 6. Manual & Automatic Operation

Cooling FAN will have

- AUTO Mode

- MANUAL Mode

Manual mode shall allow operators to independently operate field equipment for maintenance and emergency purposes.

# 7. IoT Automation Features

The Microcontroller-Based IoT Automation System shall support:

- Automatic Climate Control

- Automatic Nutrient Monitoring

- Real-Time Temperature Monitoring

- Real-Time Humidity Monitoring

- Real-Time EC Monitoring

- Real-Time pH Monitoring

- Timer-Based Irrigation

- Cyclic Timer Operations

- Manual Override

- Setpoint Configuration

- Expandable Architecture

- Remote Monitoring (future provision)

- Data Logging (future provision)

# 8. Configurable Parameters

Authorized users shall be able to configure:

- Minimum Temperature

- Medium Temperature

- Maximum Temperature

- Minimum Humidity

- Maximum Humidity

- Cooling Pad Pump ON Time

- Cooling Pad Pump OFF Time

- Fogger ON Time

- Fogger OFF Time

- ACF ON Time

- ACF OFF Time

- Overhead Sprinkler ON Time

- Overhead Sprinkler OFF Time

- EC Setpoint

- Irrigation Start Time

- Irrigation Duration

# 9. Testing & Commissioning

The scope shall include:

- Installation verification

- Input/Output testing

- Sensor calibration

- Functional testing

- Automation sequence testing

- Cyclic timer verification

- Site acceptance testing

- Performance validation

- Operator training

# 10. Documentation

InHydro shall provide:

- Automation Logic Description

- I/O List

- User Manual

# 11. Deliverables

The project deliverables shall include:

- Design and Engineering

- Microcontroller-Based IoT Automation Controller

- Electrical Control Panel

- Embedded Firmware Development

- Automation Programming

- Climate Control Logic

- Nutrient Management Logic

- Timer Programming

- Installation

- Testing & Commissioning

- Training

- Technical Documentation